# AeroSniffer User Manual

# XIAO ESP32S3 Multi-Boot Desk Gadget — DeskBuddy 2.0 Build

## 1. Introduction

AeroSniffer treats the XIAO ESP32S3 like a mini operating system with three completely separate identities. Long-press the capacitive touch sensor (1.5 seconds) to switch between a living desktop companion, a wireless security monitor, and a real-time flight radar — all running inside a DeskBuddy 2.0 enclosure.

## 2. Hardware — DeskBuddy 2.0 Kit (Rs 2,299)

Purchase from: https://www.esclabs.in/product/deskbuddy-2-0-kit/

The kit includes everything you need. No extra purchases required:

- - Seeed Studio XIAO ESP32S3 (8MB Flash, 8MB PSRAM, WiFi + BLE 5.0)

- - 1.3 inch ST7789 IPS Display (240x240 square, SPI interface)

- - Red Capacitive Touch Switch Module (digital output, active LOW)

- - 3.7V LiPo Battery (matched to enclosure)

- - On/Off Switch

- - 3D Printed Enclosure (pre-made, professionally finished)

- - USB-C Cable (data + power)

## 3. Wiring Guide (10 Wires Total)

Connect the modules to the XIAO ESP32S3 pins as follows:

ST7789 Display (SPI):

MOSI = D10 (GPIO 9)

SCLK = D8  (GPIO 7)

CS   = D3  (GPIO 4)

DC   = D2  (GPIO 3)

RST  = D9  (GPIO 8)

VCC  = 3V3,  GND = GND

BL   = Not connected (always-on on carrier board)

Capacitive Touch Module:

SIG  = D0  (GPIO 1)  — active LOW when touched

VCC  = 3V3,  GND = GND

Touch sensor placement: Mount under the roof of the 3D-printed shell. Capacitive sensing works through thin plastic (2-3mm).

## 4. Software Setup

1. Install Arduino IDE 2.x from https://www.arduino.cc/en/software

2. Add ESP32 board support URL in Preferences: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

3. Install 'esp32 by Espressif Systems' v3.0.x+ from Boards Manager.

4. Select board: XIAO_ESP32S3. Enable 'USB CDC On Boot'.

5. Install libraries: TFT_eSPI (by Bodmer), ArduinoJson (by Benoit Blanchon).

6. Copy AeroSniffer/TFT_eSPI_UserSetup.h to the TFT_eSPI library folder (User_Setup.h). It is pre-configured for ST7789 240x240 with XIAO SPI pins.

7. Edit AeroSniffer/Config.h to set your WiFi credentials and GPS bounding box.

8. Upload SPIFFS data using the LittleFS plugin (airlines.db for Mode 3).

9. Compile and Flash AeroSniffer.ino.

## 5. Operating Manual

Touch interaction:

Short tap (< 1.5s) = Context action (pet pat in Mode 1)

Long press (>= 1.5s) = Switch to next mode

Mode 1 — The Companion: Animated desk pet with expressive eyes. Tap the touch sensor to trigger the happy face. The pet blinks automatically and cycles through idle expressions.

Mode 2 — Security Monitor: The 1.3 inch screen shows an animated radar sweep with live packet statistics (PKT/s, beacons, probes, deauths). For full control, connect your phone to the 'AeroSniffer-SEC' WiFi network and open http://192.168.4.1 in a browser. The web UI lets you start/stop scans and view detailed packet data.

Mode 3 — Flight Radar: Connects to your home WiFi and polls the OpenSky Network API every 15 seconds. Displays compact flight cards with callsign, altitude, ground speed, heading, and a mini compass rose + radar map.

## 6. Companion App (Planned)

A web application (deployed on Vercel) or desktop executable is planned for Mode 2 control when the device is connected via USB. Features will include:

- - Full Marauder-style control panel via USB Serial

- - Firmware update checker and OTA capability

- - Built-in how-to tutorial and payload library

- - Real-time packet visualization and PCAP export

## 7. Troubleshooting

White screen: Check TFT_eSPI User_Setup.h was correctly replaced. Verify SPI wiring.

Upload fails: Put XIAO in download mode — Hold BOOT, press RESET, release BOOT. Ensure USB cable supports data.

No serial output: Enable 'USB CDC On Boot' in Arduino IDE board settings.

Touch not working: Verify D0 (GPIO 1) is wired to the SIG/OUT pin of the touch module.

WiFi FAILED in Mode 3: Check SSID/password are correct and router is 2.4GHz.